Verification plan objectives for Base Table >>

- Read during write (RDW) structural hazard : - Done

Need to check for the condition about what is happening when a single location in the memory is targeted for read and write at the same clock edge i.e. fetch_idx_i  == commit_idx_i and  fetch_en_i == 1  & update_en_i == 1.

Need to verify how the memory block is supposed to function during such scenario.

- Gating of clock for Setup/Hold violations :-

Need to verify if the fetch_en_i and update_en_i toggle too close to the rising edge of the clock – it can introduce a glitch. Write or reads will be silently dropped.

Need to  verify that the glitch test. Fetch_en_i = 0 and update_en_i = 0. Drive random data on inputs for 100 cycles and see what is the output – is there any toggle in output. No structural changes should occur in the data stored in SRAM.

- Read stabilization :-

Toggle fetch_en_i = 1 for one cycle to read an index, then drop it to 0. Change the fetch_idx_i value while fetch_en_i is Low. Outputs should retain the previously read values indefinitely until fecth_en_i is asserted again.

- Memory address uniqueness :- Done

Write some distinct pattern at all the indexes of the memory like distn = index, confidence = idx[2:0]  sequentially from index 0 to 1023. Then, loop back and read all 1024 rows. Each entry must perfectly return its own unique stored value. If not, then there is address bit decoding failure.

- Same cycle read and write :- Done

Set fetch_en and update_en == 1, with fetch_idx_i == update_idx_i on the exact same cycle. Need to see that the memory array successfully commits the new value at index and output port returns the previous data value held at that address.

Check the functional coverage of the design

- read enable / write enable combinations

- read addresses

- write addresses

- distance values

- confidence values

- same-cycle read/write

- same-cycle read/write same address

- same-cycle read/write different address

- low/mid/high address ranges

- boundary addresses: 0, 1, 1022, 1023

COMPLETED BASE TABLE TESTS

base_predictor_smoke_seq
Basic sanity test. Writes a few known addresses, reads them back, and checks that stored {distance, confidence} values return correctly.

base_predictor_random_seq
Fully random traffic. Randomizes read/write enables, addresses, distance, and confidence. Useful for broad stress, but many reads may hit unwritten addresses.

base_predictor_random_write_and_same_read_seq
Random write/readback test. Generates random write addresses/data, stores those addresses in a queue, then reads back exactly those same addresses. This gives useful random testing without many skipped uninitialized reads.

base_predictor_hazard
Read-after-write and same-cycle read/write hazard test. Checks:

write at cycle T, read at cycle T+1

same-cycle read/write to same address

next-cycle readback confirms new data committed

Also records whether same-address RDW returned old data, new data, or other data.

base_predictor_read_stability_seq
Read output hold test. Writes and reads one address, then keeps fetch_en_i=0. Checks that base_dist_o/base_conf_o remain stable while read is disabled.

base_predictor_unique_value_at_index_pattern_seq
Address uniqueness / decode test. Writes a distinct pattern into a range of addresses:

distance = index + 5

confidence = index[2:0]

Then reads them back to confirm each address returns its own value.

base_predictor_access_transition_seq
Access transition coverage test. Exercises back-to-back transitions:

idle -> read/write/read_write

write -> read/write/read_write

read -> read/write/read_write

read_write -> read/write/read_write

base_predictor_duplicate_write_seq
Latest-write-wins test. Writes the same address multiple times, then reads it back. Expected result is the latest written {distance, confidence}.

base_predictor_disabled_input_toggle_seq
Disabled-port toggle coverage. Keeps enables low while changing fetch/write-side inputs. This supports checking:

fetch_en=0 while fetch_idx changes

update_en=0 while commit inputs change

base_predictor_confidence_INCR_DECR_ONLY_seq
Confidence update-only test. Keeps distance unchanged at an index while changing confidence by +1 or -1, then reads back to confirm only confidence changed.

base_predictor_confidence_saturation_seq
Confidence saturation behavior test. Checks:

6 -> 7 -> 7

1 -> 0 -> 0

Distance remains unchanged while confidence saturates at max/min.

base_predictor_disabled_write_protection_seq
Disabled write protection test. Writes a known value, then drives new write-side address/data with update_en_i=0, then reads back the original address. Expected result: original value is preserved.