Verification plan for the base update control design

- Basic/smoke sequence

- Sequence with en = 0/disabled

- Correct prediction sequence with average confidence, increment in the confidence value.

- Correct predicted sequence with maximum confidence, no increment or replacement – the confidence value should remain static/saturated as is.

- Mispredictions with nonzero confidence values, confidence must decrement.

- Mispredictions with zero confidence, true distance must replace old distance

- Sweeps every confidence value for correct and mispredicted outcomes

- Test zero , maximm and alternating index/distance values.

- Alternate between enables and disabled transactions

- Sending identical consecutive transactions to verify none are missed.

- Longer constrained random run with enabled and disabled transactions.

- Drive X values on upstream inputs while en_i = 0 ; outputs must remain known and zero.

- Drive X/Z on enabe pin and need to see if the assertion fires or not.

- Run all positive directed sequences altogether.