Firstly, WHY DO WE USE (!clk) in the IF ELSE block?

It is because, we are targeting a rising edge clock and hence , we need to understand that the latched enable signal should be stable before the rising edge of the Clock signal. Hence , we took a latch sensitive to negative level sensitive clock signal.

If we would need clock gating for negative level sensitive clock then we will choose a latch sensitive to positive level of clock so that when the negative edge is about to arrive – the latched_enable signal becomes stable.

Pointers we need to test in the design ( keeping in mind that the clock gating we are aiming is for rising edge clock gating in the design – hence the clock filter will be negative level trigerred )

- Clock blocked when enable is low ->

- When en_i = 0, the latch captures 0 during the low phase of clk_i. After that, even when clk_i goes high, clk_gated_o must remain 0.

- Clock passes when enable is high

- When en_i = 1 is sampled during the low phase, en_latched becomes 1. Then during the high phase, clk_gated_o should follow clk_i.

- Enable sampled only during low clock phase

- The latch is transparent only when clk_i = 0. So en_latched should update only in the low phase and should hold its value during the high phase.

- Enable changes during high clock phase do not glitch output

- If en_i changes while clk_i = 1, en_latched should not immediately change. Therefore clk_gated_o should not glitch because of high-phase enable toggles.

- Disable takes effect after low phase

- If en_i changes from 1 to 0, the clock should not be blocked until that 0 is sampled during a low phase. After the latch captures 0, the next high phase should be blocked.

- Enable takes effect after low phase

- If en_i changes from 0 to 1, the gated clock should not pass until the latch samples the 1 during the low phase. After that, the next high phase should pass.

- Back-to-back enable patterns

- This checks repeated enable/disable transitions across consecutive cycles, like 0,1,0,1 and short enable/disable bursts. It confirms the gating logic behaves correctly under rapid but legal enable control changes.

- Random phase enable toggles

- This stresses the cell by changing en_i during low phase, high phase, and arbitrary/random times. The scoreboard reference model should still predict clk_gated_o = clk_i & ref_en_latched.