- Smoke/Basic -> Existing continuous-branch test; , all branches will be valid in the design with commit Flag and misprediction flag low.

- Startup Reset -> Verify reset values of GHR, hash folded history, FIFO pointers/count, flags, and recovery outputs.

- Reset with populated state -> Populate GHR and FIFO, then reset and verify all resettable state.

- Reset while disabled -> Assert reset with en_i=0; verify reset does not depend on functional clock enables.

- Reset during recovery -> Assert reset while recovery is pending/active; verify clean restart. (to be done at last)

- Repeated reset ->  Apply multiple reset pulses with activity between them

- Branch valid toggle -> Single-cycle and multi-cycle gaps between valid branches; check hold and resume behavior

- Global enable toggle -> Toggle en_i while br_valid_i=1; change inputs during disable and check state holds

- Enable/valid combinations -> Exercise all four combinations of en_i and br_valid_i.

- FIFO Single write -> Write one checkpoint into an empty FIFO; check empty deassertion and stored checkpoint timing

- FIFO Single read ->  Commit the only checkpoint; verify transition back to empty.

- FIFO empty to full -> Write exactly FIFO_DEPTH entries; check count and full assertion.

- FIFO write while full -> Request additional writes without reads; check agreed overflow-blocking behavior.

- FIFO drain to empty -> Fill, then commit entries until empty; check count and pointer progression.

- FIFO read while empty -> Issue commit requests on an empty FIFO; check agreed underflow-blocking behavior.

- FIFO simultaneous read/write -> Issue branch and commit together at intermediate occupancy.

- FIFO simultaneous read/write while full -> Check whether a read permits replacement of the departing entry in the same cycle

- FIFO simultaneous read/write while Empty -> Check whether the read is blocked or bypassed according to the FIFO specification

- FIFO Pointer wraparound -> Repeated writes/reads across pointer boundaries, including nonzero occupancy.

- FIFO repeated fill/drain -> Repeat full-to-empty cycles to expose accumulated count or pointer errors.

- Checkpoint data ordering -> Store distinct folds and verify checkpoint order through recovery reads or dedicated observability.

- GHR packet patterns -> Use zero, all-one, alternating, walking-one, and repeated packets.

- GHR tap depth boundaries -> Cross each history depth: 2, 4, 6, 8, 12, 16, and 32; verify all exposed expiring packets

- GHR Bank/row transaitions ->  Exercise writes and reads across bank and row boundaries.

- GHR circular wraparound -> Advance through the entire 36-entry history multiple times

- Hash fold arithematic -> Check rotation, incoming XOR, expiring alignment, and fold-width wraparound using directed patterns

- Hash PC mixing -> Hold history stable where possible and vary PC; verify index/tag calculation and ignored low PC bits

- Basic misprediction recovery -> Recover a known checkpoint and roll back the GHR with no concurrent incoming branch.

- Recovery depth sweep -> Exercise supported rollback depths using sufficiently populated history.

- Recovery across GHR wraparound -> Roll back across the circular pointer boundary

- Misprediction with empty FIFO -> Start recovery from full occupancy and verify flags and restored data.

- Misprediction with full FIFO -> Start recovery from full occupancy and verify flags and restored data

- Branch during recovery -> Check branch suppression or acceptance according to the recovery rules

- Enable low during recovery -> Disable before/during recovery, then re-enable; check whether recovery is retained and completed correctly.

- Commit and misprediction together -> (MAYBE Invalid – because a branch wont commit if mispredicted , but we will see) Verify priority and that a single request does not accidentally consume two entries.

- Branch and misprediction together ->  Verify rollback/write priority and checkpoint behavior

- Repeated misprediction requests -> Exercise adjacent requests or requests during active recovery, according to the supported protocol

- Resume after recovery -> Send known branches after recovery and verify continued GHR/fold/FIFO operation.

- Long mixed random traffic -> Mix legal branch, idle, enable, commit, and recovery operations across multiple seeds.