Verification plan for tagged update control

- Smoke sequence - Runs one representative scenario for every command, disabled behavior, counter saturation, and a few random-valid transactions. It is just a basic sequence fr all.

- Disabled/isolation sequence – Will have enable LOW, while varying normal and unknown inputs. Verifies that all outputs remain zero.

- Allocation sequence - Tests normal allocation, initialization of confidence/usefulness, replaceable-entry requirements, and tag/distance boundaries.

- Reward sequence - Tests normal increments, confidence saturation, usefulness saturation, dual saturation, and preservation of tag/distance.

- Penalize sequence - Tests confidence decrement, confidence one-to-zero, usefulness preservation, and preservation of the current tag and distance.

- Zero-confidence retraining sequence - Penalizes an entry already at zero confidence and verifies distance replacement, weak-confidence restoration, and tag/usefulness preservation.

- Decay sequence - Tests usefulness decrement, zero saturation, maximum usefulness, and preservation of confidence, tag, and distance.

- Counter sweep sequence - Sweeps all confidence and usefulness values across reward, penalize, and decay operations.

- Data boundary sequence- Exercise zero, maximum, alternating-bit, and representative tag and distance values.

- Repeated reward sequence - Applies consecutive rewards until confidence and usefulness reach saturation.

- Repeated penalize sequence - Applies consecutive penalties until confidence reaches zero, then performs distance retraining.

- Entry lifecycle sequence - Runs allocation, reward, penalize, decay-to-zero, and reallocation while carrying each result into the next transaction.

- Random valid sequence - Generates randomized commands while maintaining the required command-specific input relationships.

- Invalid protocol sequence - Deliberately violates allocation, reward, and penalize input requirements and injects unknown command/control values; scoreboard errors are expected.